National Textile University, Faisalabad

Department of Computer Science

| Name: | Muhammad Ali Akram |
| --- | --- |
| Class: | BSCS-5th-B |
| Registration No: | 23-NTU-CS-1062 |
| Course Name: | Embedded IoT |
| Submitted To: | Sir Nasir |
| Submission Date: | October 26, 2025 |

# Code:

#include <Arduino.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

// =================== OLED SETUP ===================

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_ADDR 0x3C

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// =================== TASK A PINS ===================

#define LED1        19     // LED 1

#define LED2        18     // LED 2

#define BTN_MODE    26     // Button to cycle modes

#define BTN_RESET   0      // Onboard BOOT button (reset to OFF)

// =================== TASK B PINS ===================

#define LED3        17     // Single LED for Task B

#define BTN_EVENT   25     // Button for short/long press

#define BUZZER_PIN  32     // Buzzer pin

// =================== PWM SETTINGS ===================

#define PWM_CH_LED1   0

#define PWM_CH_LED2   1

#define PWM_CH_BUZZER 2

#define PWM_FREQ      5000

#define PWM_RES       10    // 10-bit (0–1023)

// =================== VARIABLES ===================

// ---- Task A ----

int mode = 0; // 0=OFF, 1=Blink, 2=ON, 3=Fade

bool ledState = false;

unsigned long prevMillisBlink = 0;

int fadeVal = 0;

int fadeDir = 10;

unsigned long lastDebounce = 0;

const int debounceDelay = 300;

// ---- Task B ----

bool pressed = false;

unsigned long pressStart = 0;

bool led3State = false;

bool buzzerActive = false;

// =================== OLED FUNCTION ===================

void showOLED(const String &line1, const String &line2 = "") {

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 5);

display.print(line1);

display.setTextSize(1);

display.setCursor(0, 40);

display.print(line2);

display.display();

}

// =================== SETUP ===================

void setup() {

Serial.begin(115200);

// ---- OLED ----

Wire.begin(21, 22);

if (!display.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR)) {

Serial.println("OLED init failed!");

for (;;);

}

showOLED("System", "Initializing...");

delay(1000);

// ---- LEDs and Buttons ----

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

pinMode(LED3, OUTPUT);

pinMode(BTN_MODE, INPUT_PULLUP);

pinMode(BTN_RESET, INPUT_PULLUP);

pinMode(BTN_EVENT, INPUT_PULLUP);

// ---- PWM Setup ----

ledcSetup(PWM_CH_LED1, PWM_FREQ, PWM_RES);

ledcSetup(PWM_CH_LED2, PWM_FREQ, PWM_RES);

ledcSetup(PWM_CH_BUZZER, PWM_FREQ, PWM_RES);

ledcAttachPin(LED1, PWM_CH_LED1);

ledcAttachPin(LED2, PWM_CH_LED2);

ledcAttachPin(BUZZER_PIN, PWM_CH_BUZZER);

// ---- Start in Mode 0 (OFF) ----

ledcWrite(PWM_CH_LED1, 0);

ledcWrite(PWM_CH_LED2, 0);

showOLED("Mode 1", "Both OFF");

}

// =================== LOOP ===================

void loop() {

unsigned long now = millis();

// ===================== TASK A =====================

bool btnMode = digitalRead(BTN_MODE);

bool btnReset = digitalRead(BTN_RESET);

// ---- Mode Button (Cycle) ----

if (btnMode == LOW && (now - lastDebounce > debounceDelay)) {

lastDebounce = now;

mode = (mode + 1) % 4;

switch (mode) {

case 0: showOLED("Mode 1", "Both OFF"); break;

case 1: showOLED("Mode 2", "Alt Blink"); break;

case 2: showOLED("Mode 3", "Both ON"); break;

case 3: showOLED("Mode 4", "PWM Fade"); break;

}

}

// ---- Reset Button (BOOT) ----

if (btnReset == LOW && (now - lastDebounce > debounceDelay)) {

lastDebounce = now;

mode = 0;

showOLED("Mode 1", "Reset to OFF");

}

// ---- LED Control ----

switch (mode) {

case 0: // Both OFF

ledcWrite(PWM_CH_LED1, 0);

ledcWrite(PWM_CH_LED2, 0);

break;

case 1: // Alternate blink

if (now - prevMillisBlink >= 500) {

prevMillisBlink = now;

ledState = !ledState;

ledcWrite(PWM_CH_LED1, ledState ? 512 : 0);

ledcWrite(PWM_CH_LED2, ledState ? 0 : 512);

}

break;

case 2: // Both ON

ledcWrite(PWM_CH_LED1, 512);

ledcWrite(PWM_CH_LED2, 512);

break;

case 3: // PWM fade

ledcWrite(PWM_CH_LED1, fadeVal);

ledcWrite(PWM_CH_LED2, fadeVal);

fadeVal += fadeDir;

if (fadeVal <= 0 || fadeVal >= 1023) fadeDir = -fadeDir;

delay(5);

break;

}

// ===================== TASK B =====================

bool btnEvent = digitalRead(BTN_EVENT);

if (btnEvent == LOW && !pressed) {

pressed = true;

pressStart = now;

buzzerActive = false;

}

if (pressed && btnEvent == LOW) {

// Button is being held

unsigned long holdTime = now - pressStart;

if (holdTime > 1500 && !buzzerActive) {

// ---- Long press detected (while holding) ----

showOLED("Long Press", "Buzzer Tone");

ledcWriteTone(PWM_CH_BUZZER, 1000);

buzzerActive = true;

Serial.println("Long press started - buzzer active");

}

}

if (btnEvent == HIGH && pressed) {

// Button released

pressed = false;

if (!buzzerActive) {

// ---- Short press ----

led3State = !led3State;

digitalWrite(LED3, led3State);

showOLED("ShortPress", led3State ? "LED3 ON" : "LED3 OFF"); // fixed wrapping

Serial.println("Short press detected.");

} else {

// ---- Stop buzzer when released ----

ledcWrite(PWM_CH_BUZZER, 0);

buzzerActive = false;

Serial.println("Long press released - buzzer stopped");

}

}

}

# Pin Map:

| Component | Pin (GPIO) |
| --- | --- |
| OLED Display | SDA: 21 / SCL: 22 |
| Task A |  |
| LED 1 (Cyan) | 19 |
| LED 2 (Green) | 18 |
| Mode Button | 26 |
| Reset Button | 0 (On-Board BOOT Button) |
| Task B |  |
| LED 3 | 17 |
| Event Button | 25 |
| Buzzer | 32 |

# Screenshots:

# Initializing:

# Mode 1:

# Mode 2:

# Mode 3:

# Mode 4:

# Reset (Boot) Button:

# Short Press OFF:

# Short Press ON:

# Long Press (>1.5 sec):